# Building Access Survey

121 East 7th Place, Suite 107

St. Paul, MN 55101

651-361-7800 or 1-800-945-8913

www.disability.state.mn.us

Table of Contents

PLANNING INTRODUCTION	4

Planning Requirements for Different Facilities	4

PLANNING ANALYSIS	5

Step By Step	5

Site	6

Building	6

Spaces	8

Elements	8

Special Facilities	8

SURVEY FORMS	8

Building Access Survey – Cover Sheet	9

Planning Summary Sheet	10

SITES	12

Parking Space Requirements	12

Parking Survey Form	13

Drop-Off Zone	15

Walkways	16

Ramps	17

Curb Ramp	19

Steps (It is suggested that all stairs meet this criteria.)	20

BUILDING	21

Stair (It is suggested that all stairs meet this criteria.)	21

Lift	22

Elevator	23

Ramp	25

Doorway	27

Corridor	32

SPACES	34

Toilet Room	34

Bathtub	39

Shower	41

Break Room	44

Auditorium	45

Conference Room	46

Dressing and Fitting Room	48

ELEMENTS	49

Telephone	49

Drinking Fountain	50

Information Kiosk and Automated Teller Machine	51

Sales and Service Counters	52

SPECIAL FACILITIES	54

Restaurant and Cafeteria	54

Library	56

Medical Care Facility	57

Transient Lodging	59

Children’s Environments	63

Some of the information in this document was developed by the Adaptive Environments Center in cooperation with the Massachusetts Executive Office of Administration and Finance Division of Capital Planning and Operations and the Office of Disability Affairs. Based on Code Revisions of 2007, updated in 2015.

# PLANNING INTRODUCTION

The Planning Section has been designed to make to make the surveying task more manageable; it will help you prepare for surveying the facility. You can determine ahead of time how much work is involved and whether you will need others to help you. Specifically, the planning section allows you to determine – before you leave your office – how many Survey Forms you will need to copy and take with you.

For each building that you evaluate you will be looking at four different areas of the building and the land around it. Each area has several subcomponents. There may be additional accessibility requirements based on the type of facility being reviewed. Your building may have special requirements based on the use and type of facility. Look under Special Facilities to determine if your building has additional requirements.

## PLANNING REQUIREMENTS FOR DIFFERENT FACILITIES

### Site

Parking, Drop-off, Walkway, Ramp, Curb Ramp, Steps

### Building

Stair, Lift, Elevator, Ramp, Doorway, Corridor

### Spaces

Toilet Room, Bathtub, Shower, Break Room, Auditorium, Conference Room, Dressing & Fitting, Room

### Elements

Telephone, Drinking Fountain, Kiosk & ATM, Sales & Service Counters

### Special Facilities

Restaurant & Cafeteria, Library, Medical Care Facilities, Transient Lodging, Children’s Environments

# PLANNING ANALYSIS

What needs to be accessible at my facility? This section will help you analyze the four critical areas of your building/facility: Site, Building, Spaces, Elements, and possibly, Special Facilities.

Read the entire Planning section carefully to determine what portion of your facility must be accessible. At the same time, note (on the Planning Summary Sheet) what Survey Forms you will need for completing the field survey portion of the process.  If you have more than one building at your facility, you will need to plan for each one individually.

## STEP BY STEP

Identify team members. These individuals will provide guidance and assistance during the planning, surveying, and reviewing steps. The team might include the facility manager, chief administrator, maintenance supervisor, 504 Coordinator or Equal Opportunity officer, financial staff, and persons with disabilities.

Fill in the Planning Summary Sheet. For each building to be surveyed, read through the entire Planning section and mark on the Planning Summary Sheet the total number of Survey Forms that will be needed. If you have more than two buildings at your facility, use additional Planning Summary Sheets.

Copy the Survey Forms. You will not need to use all of the survey forms provided in this booklet. Photocopy the necessary survey forms for the facility being reviewed. Keep this survey booklet as a reference and for future surveying needs. Be sure to copy an extra set of forms to have handy when surveying (in the event an unexpected barrier is encountered). Collate and staple the appropriate forms on a building-by-building basis. For each survey form, fill out each and every question. It is very important that all issues are addressed and that an answer is provided. If a particular question is not applicable, then note this by checking “N/A”. This will indicate that the question has not been overlooked or forgotten. The importance of this will become clear when the survey is reviewed at a later date. A comment section is provided at the end of each survey form. Use this area for notes and to clarify special situations which may occur.

## SITE

Site accessibility involves arriving at the site, parking a vehicle or being dropped off, and getting to a building or outdoor recreation area. It also includes the ability to move from one building to another when there is more than one building at a facility.

People with mobility issues who arrive by vehicle need to be able to enter buildings on their own – independently – without assistance from others. Direct and safe walkways from these areas as well as from the street and transportation stops are essential for people with mobility and sight impairments.

There are six possible elements to site accessibility: PARKING, DROP-OFF ZONE, WALKWAY, RAMP, CURB RAMP, and STEPS. Determine which elements are present or should be provided at the facility being reviewed and enter the number of survey forms needed on the Planning Summary Sheet.

## BUILDING

Once an accessible route has been provided to the building, an accessible entrance is essential to making a building usable by people with disabilities. As many entrances as possible should be accessible, especially the entrance used most often by the nondisabled public. Asking individuals with disabilities to use basement or back doors not used by others not only discriminates against them but also puts them at a disadvantage by depriving them of services provided in a front lobby: signage, reception, and waiting areas. It is important that once someone is inside the entrance that they be able to easily gain direct access to elevators and corridors that lead to other parts of the building.

Sixty percent of primary public entrances to a building are required to be accessible. As many employee and service entrances as possible should also be accessible. For example, in large buildings such as hospitals there may be an emergency entrance, a visitor entrance, and an entrance that staff use located close to where they park their cars. In this instance, all three entrances should be accessible. It is often the case that parking lots are located at the back of the buildings. If the main entrance is located on a public street but a long walk from the back of the building, both entrances should be made accessible.
Some criteria for determining which entrances should be accessible include:

It is referred to as the “main entrance” to the building. (If you asked for directions to this building, you would be directed to this entrance.)

It provides the most direct access to main corridors and elevators (if present) as well as major public function spaces at the entry level such as an auditorium or cafeteria.

It is an entrance that people use when they enter the building from visitor or staff parking areas.

Accessible routes within and throughout the building or facility are the next area which need to be considered in order to provide access for persons with disabilities. Changes in elevation can be accomplished by ramp, elevator or lift.

People often think that getting around within a building is only a problem for someone using a wheelchair. Individuals with limited or no vision find it difficult to use buildings with poor signage, obstructions in the hallways, and elevators without audible signals. People with leg braces or prosthesis find poorly designed stairs difficult, if not impossible, to use. It is important to remember that even in elevator buildings some stairways are major connections between floors and therefore should be accessible.

There are six key components to entering and/or getting around within the building: STAIR, LIFTS, ELEVATORS, RAMP, DOORWAY, CORRIDORS. You may find that some elements are repeated or duplicated, i.e. there may be several sets of elevators in a building but they are all identical, you would only need to survey one of the elevators in this case. Determine which elements are present or should be provided at the facility being reviewed and enter the number of survey forms needed on the Planning Summary Sheet.

## SPACES

Once you have made it possible for people to move about the facility easily, you need to determine what spaces should be made accessible. Even if there are only a few people with disabilities currently using a building, there will eventually be more. Remember that individuals with disabilities are not only visitors and clients, but they are also employees and management personnel.

Rooms like toilet rooms will be found in all buildings. Other spaces are more specialized and occur less frequently but must be accessible because of their public use. Examples of special spaces include break rooms, auditoriums, and conference rooms. Survey forms are provided for the TOILET ROOM, BATHTUB, SHOWER, BREAK ROOM, AUDITORIUM, CONFERENCE ROOM, DRESSING & FITTING ROOM. Determine which elements are present or should be provided at the facility being reviewed and enter the number of survey forms needed on the Planning Summary Sheet.

## ELEMENTS

Elements such as telephones, drinking fountains, ATM machines, and service counters are sometimes provided in the building or facility. These elements must also meet accessibility requirements. There are survey forms to be completed for each of these elements and if, for example, there is a bank of public telephones in the lobby and also a bank of telephones adjacent to a conference area, both banks must be surveyed. Determine which elements are present or should be provided at the facility being reviewed and enter the number of survey forms needed on the Planning Summary Sheet.

## SPECIAL FACILITIES

Some buildings are designed for special use and purposes. There are specific accessibility requirements developed for certain types of facilities. The accessibility requirements for these facilities are in addition to the requirements for other buildings. Use the survey forms provided in the previous sections for the “generic” areas in special use facilities. The survey forms provided in this section present accessibility requirements that go beyond the general requirements and are specific to a particular type of facility.

Special Facilities include: LIBRARY, RESTAURANT & CAFETERIA, MEDICAL CARE FACILITY, TRANSIENT LODGING, and CHILDREN’S ENVIRONMENTS.

# SURVEY FORMS

## BUILDING ACCESS SURVEY – COVER SHEET

FILL OUT FOR EACH BUILDING

Note to screen reader users: use the arrow keys to navigate these forms.

BUILDING:

DATE OF SURVEY:

STREET ADDRESS:

CITY/STATE:

BUILDING CONTACT NAME:

BUILDING CONTACT INFORMATION:

SURVEYOR’S NAME:

SURVEYOR’S CONTACT INFORMATION:

SURVEYOR’S POSITION OR TITLE:

## PLANNING SUMMARY SHEET

Facility:      	Completed by:

Total number of buildings:      	Completion date:

This form is used to identify the specific number of Survey Forms you will need to package for each building at your facility. Fill it out as you become acquainted with the layout of each building being surveyed. After it is completed, photocopy the total number of each Survey Forms that you will need in the field and then collate the forms on a

building-by-building basis. (If more than one building, make copies and use additional Planning Summary Sheets.)

LIST THE NUMBER OF SURVEY FORMS NEEDED AND ANY COMMENTS, BELOW

### BUILDING 1

Parking	   	Comments

Drop-Off Zone   	Comments

Walkway	   	Comments

Ramp	   	Comments

Curb Ramp	   	Comments

Steps	   	Comments

Stairs	   	Comments

Lift	   	Comments

Elevator	   	Comments

Doorway	   	Comments

Corridor	   	Comments

Toilet Room Male	   	Toilet Room Female	   	Toilet Room Unisex	   	Comments

Bathtub Male	   	Bathtub Female	   	Bathtub Unisex	   	Comments

Shower Male	   	Shower Female	   	Shower Unisex	   	Comments

Dressing Room Male	   	Dressing Room Female	   	Dressing Room Unisex	   	Comments

(Cont. from page 10)

Break Room	   	Comments

Auditorium	   	Comments

Conference Room	   	Comments

Telephone	   	Comments

Kiosk	   	Comments

Sales & Service Counter	   	Comments

Drinking Fountain	   	Comments

ATM	   	Comments

Restaurant	   	Comments

Library	   	Comments

Transient Lodging	   	Comments

Cafeteria	   	Comments

Medical Facility	   	Comments

Children’s Environment	   	Comments

## SITES

### PARKING SPACE REQUIREMENTS

For 1 to 25 parking spaces, 1 Accessible Spaces Required, 1 of which must be Van Accessible

For 26 to 50 parking spaces, 2 Accessible Spaces Required, 1 of which must be Van Accessible

For 51 to 75 parking spaces, 3 Accessible Spaces Required, 1 of which must be Van Accessible

For 76 to 100 parking spaces, 4 Accessible Spaces Required, 1 of which must be Van Accessible

For 101 to 150 parking spaces, 5 Accessible Spaces Required, 1 of which must be Van Accessible

For 151 to 200 parking spaces, 6 Accessible Spaces Required, 1 of which must be Van Accessible

For 201 to 300 parking spaces, 7 Accessible Spaces Required, 2 of which must be Van Accessible

For 301 to 400 parking spaces, 8 Accessible Spaces Required, 2 of which must be Van Accessible

For 401 to 500 parking spaces, 9 Accessible Spaces Required, 2 of which must be Van Accessible

For 501 to 1000 parking spaces, 2% of Total Accessible Spaces Required, of which 1 in every 6, or fraction thereof, must be Van Accessible

For 1000+ parking spaces, 20 plus 1 for each additional 100 Accessible Spaces Required, of which 1 of 6, or fraction thereof, must be Van Accessible

*NOTE: “van accessible” spaces are included in the number of “accessible spaces required.”

One in every six accessible spaces must be van accessible, having a minimum vertical clearance of 98 inches.

### PARKING SURVEY FORM

Are there accessible spaces provided per Parking Spaces Requirements?	 Yes  No  N/A

Are parking spaces 8 feet wide with an adjacent 8 foot wide access aisle? (Parking spaces can share an access aisle.) 	 Yes  No  N/A

- Are access aisles identified with a sign stating “no parking” centered at the head end of the space no more than 8 feet from the space?	 Yes  No  N/A

- OR

- Only where the sign would obstruct the pedestrian route, is the access aisle marked with “no parking” on the surface of the access aisle?	 Yes  No  N/A

Where the access aisle is marked with a sign, is the bottom edge of the sign between 60 inches and 66 inches above the surface of the access aisle? 	 Yes  No  N/A

Does the van accessible space have a minimum clear height of 98 inches at the space and along the vehicular route leading to the space?	 Yes  No  N/A

If all spaces do not have a clear height of 98 inches, do the van spaces have a sign indicating “van accessible”?

- Yes  No  N/A

Are the access aisles connected directly to the accessible route leading to the building entrance?

- Yes No  N/A

Are curb ramps provided along the accessible route where necessary?  (e.g., connecting the access aisle to sidewalk)	 Yes  No  N/A

Is each accessible space identified with a sign displaying the international wheelchair symbol, indicate that a permit is required, and that there is a maximum $200 fine for violation?	 Yes  No  N/A

Is the lettering and international wheelchair symbol on the sign white with a blue background?

- Yes  No  N/A

Is the sign centered at the head end of the space no more than 8 feet from the space?	 Yes  No  N/A

- OR

- If parallel parking, is the sign located on the side of the space at the head end?	 Yes  No  N/A

Is the bottom edge of the sign between 60 inches and 66 inches above the surface of the parking lot?

- Yes  No  N/A

Are the designated spaces located as close as possible to an accessible entrance?	 Yes  No  N/A

For facilities with more than one accessible entrance, are the designated parking spaces dispersed among the various accessible entrances?	 Yes  No  N/A

Is the parking area and access aisle a firm, stable, slip-resistant surface with a slope of no more than 1:48 (2%) in all directions?	 Yes  No  N/A

### DROP-OFF ZONE

- Does the drop-off area provide an access aisle 5 feet wide by 20 feet long adjacent and parallel to the vehicle pull-up space?	 Yes  No  N/A

Does the drop-off area provide a minimum clear height of at least 114 inches at the space and along the vehicular route leading to the space?	 Yes  No  N/A

Is the parking area and access aisle a firm, stable, slip-resistant surface with a slope of no more than 1:48 (2%) in all directions?	 Yes  No  N/A

Is the access aisle connected directly to the accessible route leading to the building entrance?	 Yes  No  N/A

Are curb ramps provided along the accessible route where necessary (i.e. along the accessible route from parking, drop-off, or sidewalk)?	 Yes  No  N/A

Is the drop-off area located as close as possible to an accessible entrance?	 Yes  No  N/A

### WALKWAYS

(This section only applies to exterior walkways connecting accessible parking spaces and accessible drop-off areas to accessible building entrances and to exterior walkways connecting buildings that are on the same site.)

- Is the walkway at least 4 feet wide?	 Yes  No  N/A

Is the walkway sloped a maximum of 1:20 (5%) in the direction of travel?	 Yes  No  N/A

Is the cross slope a maximum of 1:48 (2%)?	 Yes  No  N/A

Does the walkway have a firm, stable, slip-resistant surface?	 Yes  No  N/A

Do all changes in level between ¼ inch and ½ inch have beveled edges (Changes greater than ½ inch are not allowed.)	 Yes  No  N/A

If gratings are located within the walkway, do they have spaces no greater than ½ inch wide with the long dimension perpendicular to the direction of travel?	 Yes  No  N/A

Are curb ramps provided along the accessible route where necessary (i.e. along the accessible route from parking, drop-off, or sidewalk)?	 Yes  No  N/A

### RAMP

(This section does not apply to exterior walkways connecting accessible parking spaces and accessible drop-off areas to accessible building entrances or to exterior walkways connecting buildings that are on the same site. Use the SITE: WALKWAYS section for those routes.)

- Is the ramp a minimum of 36 inches wide?	 Yes  No  N/A

Is the maximum slope no greater than 1:12 (8.3%)?	 Yes  No  N/A

Is the cross slope no greater than 1:48 (2%)?	 Yes  No  N/A

Is the surface of the ramp firm, stable, and slip-resistant?	 Yes  No  N/A

Is there a 5-foot landing, measured in the direction of the ramp, at top and bottom of the ramp?

- Yes No  N/A

If the total rise exceeds 30 inches, are there intermediate landings at least 5 feet in length, located so that no ramp segment has a rise greater than 30 inches?	 Yes  No  N/A

If the ramp changes direction at the landing, is a turning space provided on the landing (generally a 5-foot diameter circle)?	 Yes  No  N/A

If the total rise exceeds 6 inches, are there handrails provided on both sides of the ramp?	 Yes  No  N/A

- Are the handrails mounted between 34 inches and 38 inches above the ramp surface?	 Yes  No  N/A

Do handrails extend horizontally 12 inches beyond the top and bottom of the ramp?	 Yes  No  N/A

Is the space between the handrail and the wall a minimum of 1 ½ inches?	 Yes  No  N/A

Are the ends of handrails looped or returned to wall, floor, or post?	 Yes  No  N/A

Do ramps and landings with drop-offs have curbs, walls, or railings which prevent a wheelchair from going over the edge? (Curbs must be at least 2 inches high.)	 Yes  No  N/A

### CURB RAMP

- Is the slope of the curb ramp a maximum of 1:12 (8.3%) measured in the direction of travel?	 Yes  No  N/A

Is the curb ramp a minimum of 36 inches wide, exclusive of the flared edges?	 Yes  No  N/A

Is the transition from the curb ramp to the adjoining surface flush and free of abrupt changes?

- Yes  No  N/A

Is the slope of the surface immediately adjoining the curb ramp a maximum of 1:20 (5%)?	 Yes  No  N/A

Is there a landing at the top of the curb ramp at least 3 feet measured in the direction of travel?

- Yes  No  N/A

### STEPS (It is suggested that all stairs meet this criteria.)

- Are riser heights uniform throughout the entire stair run?	 Yes  No  N/A

Are tread depths uniform throughout the entire stair run?	 Yes  No  N/A

Is the tread at least 11 inches in depth?	 Yes  No  N/A

Are risers closed with a maximum nosing of 1 ½ inch?	 Yes  No  N/A

Are undersides of nosings curved so that there are no abrupt edges or lips?	 Yes  No  N/A

Are handrails provided on both sides of the stairway? (On switchback steps, inside handrail shall be continuous.)

- Yes  No  N/A

Is the space between the inside edge of the railing and the wall a minimum of 1½ in?	 Yes  No  N/A

Are handrails mounted between 34 inches and 38 inches above the stair nosing?	 Yes  No  N/A

Do handrails extend horizontally 12 inches beyond the top riser?	 Yes  No  N/A

Do bottom handrails continue to slope a distance equal to the depth of one tread past bottom riser?

- Yes  No  N/A

Are the ends of handrails looped or returned to wall, floor, or post?	 Yes  No  N/A

## BUILDING

### STAIR (It is suggested that all stairs meet this criteria.)

NOTE: See SITE: STEPS illustrations on previous page for reference.

- Are riser heights uniform throughout the entire stair run?	 Yes  No  N/A

Are tread depths uniform throughout the entire stair run?	 Yes  No  N/A

Is the tread at least 11 inches in depth?	 Yes  No  N/A

Are risers closed with a maximum nosing of 1 ½ inch?	 Yes  No  N/A

Are undersides of nosings curved so that there are no abrupt edges or lips?	 Yes  No  N/A

Are handrails provided on both sides of the stairway? (On switchback steps, inside handrail shall be continuous.)		 Yes  No  N/A

Is the space between the inside edge of the railing and the wall a minimum of 1½ inches?	 Yes  No  N/A

Are handrails mounted between 34 inches and 38 inches above the stair nosing?	 Yes  No  N/A

Do handrails extend horizontally 12 inches beyond the top riser?	 Yes  No  N/A

Do bottom handrails continue to slope a distance equal to the depth of one tread past bottom riser?

- Yes  No  N/A

Are the ends of handrails looped or returned to wall, floor, or post?	 Yes  No  N/A

### LIFT

- Does the lift have a maximum vertical travel of 14 feet?	 Yes  No  N/A

Does the lift provide independent entry, operation, and exit such that a person can operate the lift without assistance?	 Yes  No  N/A

If a single door or doors on opposite ends, is the size of the platform at least 32 inches wide by 48 inches long?		 Yes  No  N/A

If a single door or doors on opposite ends, is the size of the platform at least 32 inches wide by 48 inches long?		 Yes  No  N/A

- OR

- If doors on adjacent sides, is the size of the platform at least 42 inches wide and a total area of 18 square feet?		 Yes  No  N/A

Do operating controls require no more than 5 lbs. of force to activate?	 Yes  No  N/A

### ELEVATOR

- Are the hall call buttons a maximum of 48 inches above the floor?	 Yes  No  N/A

Are visual and audible signals provided to indicate when the elevator has arrived?	 Yes  No  N/A

Does the audible signal sound once for the up direction and twice for the down direction?	 Yes  No  N/A

Are the car control buttons a maximum of 48 inches above the floor, or 54 inches in elevators installed prior to 2007 or elevators with more than 16 stops?	 Yes  No  N/A

Are hall call buttons a minimum of ¾ inch in diameter?	 Yes  No  N/A

Are control buttons a minimum of ¾ inch in diameter?	 Yes  No  N/A

Are there raised and Braille numbers and/or characters to the left of each control button?	 Yes  No  N/A

Do the hoistway entrances have raised and Braille numbers on both jambs?	 Yes  No  N/A

Do the minimum inside car dimensions comply with the diagrams below?	 Yes  No  N/A

Does the emergency communication device provide both audible and visual communication?	 Yes  No  N/A

### RAMP

- Is the ramp a minimum of 36 inches wide (measured between handrails)?	 Yes  No  N/A

Is the maximum slope no greater than 1:12 (8.3%)?	 Yes  No  N/A

Is the cross slope no greater than 1:48 (2%)?	 Yes  No  N/A

Is the surface of the ramp firm, stable, and slip-resistant?	 Yes No  N/A

Is there a 5-foot landing, measured in the direction of the ramp, at the top and bottom of the ramp?

- Yes  No  N/A

If the total rise exceeds 30 inches, is there a 5 foot intermediate landing located no more than 30 inches above the bottom of the ramp?	 Yes  No  N/A

If the ramp changes direction at the landing, is a turning space provided on the landing (generally a 5 foot diameter circle)?	 Yes  No  N/A

If the total rise exceeds 6 inches, are there handrails provided on both sides of the ramp?	 Yes  No  N/A

- Are the handrails mounted between 34 inches and 38 inches above the ramp surface?	 Yes  No  N/A

Do handrails extend horizontally 12 inches beyond the top and bottom of the ramp?	 Yes  No  N/A

Is the space between the handrail and the wall a minimum of 1 ½ inches?	 Yes  No  N/A

Are the ends of handrails looped or returned to wall, floor, or post?	 Yes  No  N/A

Do ramps and landings with drop-offs have curbs, walls, or railing which prevent a wheelchair from going over the edge? (Curbs must be at least 4 inches high.)	 Yes  No  N/A

### DOORWAY

- With the door in a 90 degree open position, is there a minimum of 32 inches of clear space from the face of the door to the latch side doorstop? (Exception: doors not requiring passage, such as shallow closets, may have clear openings of 20 inches minimum.)	 Yes  No  N/A

If a doorway has two independently operated door leaves, does at least one leaf provide the 32 inch clear space?

- Yes  No  N/A

If there are two sets of doors in a series, as in a vestibule, is there a minimum distance between the doors of 4 feet plus the width of the in-swinging door?	 Yes  No  N/A

If there are two sets of doors in a series, as in a vestibule, is there a turning space between the doors? (Generally a 5 foot diameter circle.)	 Yes  No  N/A

Is the threshold no more than ½ inch in height?	 Yes  No  N/A

Is the door hardware operable by a single effort with one hand not requiring tight grasping, pinching or twisting of the wrist? (such as a lever or loop style?)	 Yes  No  N/A

Is the threshold no more than ½ inch in height?	 Yes  No  N/A

Is the door hardware mounted no more than 48 inches above the floor?	 Yes  No  N/A

For interior doors, is the force required to open the door no more than 5 lbs.? (Does not apply to “fire” doors.)		 Yes  No  N/A

Is there a minimum of 18 inches of clear space on the latch side from the pull side of the door? (Not required for doors with automatic opener.)	 Yes  No  N/A

If the door has both a closer and a latch, is there a minimum of 12 inches of clear space on the latch side from the push side of the door? (Exceptions: doors with automatic opener.)	 Yes  No  N/A

- Do other doors have clearances as required in diagrams above? (not required for doors with automatic opener.)		 Yes  No  N/A

If the door has an automatic push button control, is the push button control mounted no more than 48 inches above the floor? (prefer 30 to 36 inches above the floor)	 Yes  No  N/A

### CORRIDOR

- Are all corridors a minimum of 36 inches wide?	 Yes  No  N/A

If there are portions of the corridor less than 36 inches wide, are they a minimum of 32 inches wide with a maximum distance of 24 inches?	 Yes  No  N/A

If the corridor is less than 5 feet wide, are there passing spaces at least 5 feet by 5 feet located at intervals not exceeding 200 feet?	 Yes  No  N/A

If there are level changes of the floor between ¼ inch and ½ inch in height, are they beveled? (Changes up to ¼ inch do not require a beveled edge.)	 Yes  No  N/A

Are all levels of the floor which are more than ½ inch above the main floor accessible by ramp, lift, or elevator? 		 Yes  No  N/A

If informational signage is provided, is it mounted on the wall on the latch side of the door between 40 inches and 70 inches above the floor?	 Yes  No  N/A

If signs have Braille, are Braille cells mounted between 48 inches and 60 inches above the floor?

- Yes  No  N/A

Do signs indicating restrooms, room numbers and names, and exit stairways have both raised characters and Braille?	 Yes  No  N/A

Are characters, symbols, and background of signs eggshell, matte, or other non-glare finish?	 Yes  No  N/A

Do characters and symbols contrast significantly in color with their backgrounds?	 Yes  No  N/A

Do public use counters have a section of the counter that is at least 36 inches wide and no more than 36 inches above the floor?	 Yes  No  N/A

Do objects protruding from walls between 27 inches and 80 inches above the floor project no more than 4 inches from the wall?	 Yes  No  N/A

If provided, do alarms have both audible and visual signals?	 Yes  No  N/A

## SPACES

### TOILET ROOM

(please fill out one assessment sheet for each Toilet Room unless they are identical to each other)

Male 	Female 	Unisex 	Floor or Building

- With entry door in a 90 degree open position, is there a minimum of 32 inches of clear space from the face of the door to the latch side door stop?	 Yes  No  N/A

If there are two sets of doors in a series, as in a vestibule, is there a minimum distance between the doors of 4 feet plus the width of the in-swinging door?	 Yes  No  N/A

If there are two sets of doors in a series, as in a vestibule, is there a turning space between the doors (generally a 5 foot diameter circle)?	 Yes  No  N/A

Is the toilet bowl centered between 16 inches and 18 inches from a side wall?	 Yes  No  N/A

Is the toilet area, measured from the wall next to the toilet, a minimum of 60 inches wide?	 Yes  No  N/A

Does the toilet area have a minimum of 48 inches of clear floor space from the front of the toilet bowl to the compartment/room wall?	 Yes  No  N/A

- OR

- Is the depth of the space at least 78 inches measured from the wall behind the toilet?	 Yes  No  N/A

Is the flush valve located on the wide side of the toilet area mounted no more than 48 inches above the floor? (If mounted above the grab bar, the flush valve must be at least 12 inches above the grab bar but no more than 48 inches above the floor.)	 Yes  No  N/A

Is the height of the toilet seat between 17 inches and 19 inches above the floor?	 Yes  No  N/A

Does the compartment door have a minimum clear opening of 32 inches?	 Yes  No  N/A

Is the compartment or room, door-locking hardware easy to operate without requiring tight grabbing, pinching or twisting?	 Yes  No  N/A

Are both horizontal and vertical grab bars provided as shown in diagrams on the next page?	 Yes  No  N/A

Is the toilet paper dispenser mounted below the horizontal grab bar as shown in diagram?	 Yes  No  N/A

If there are 2 or more toilet compartments in a room, is there an ambulatory accessible compartment measuring 36 inches wide provided in addition to the wheelchair accessible compartment?	 Yes  No  N/A

If there are two or more urinals, is at least one urinal mounted with the rim no more than 17 inches above the floor?	 Yes  No  N/A

Is the rim of the sink a maximum height of 34 inches above the floor?	 Yes  No  N/A

Is there a minimum of 27 inches of knee clearance from the floor to the bottom of the apron or counter?

- Yes  No  N/A

Does the knee clearance extend at least 8 inches back from the front edge?	 Yes  No  N/A

Is there a toe clearance that extends at least 17 inches back from the front edge at a height of 9 inches above the floor?	 Yes  No  N/A

Are the knee and toe clearances at least 30 inches in width?	 Yes  No  N/A

Do the faucets have lever handles or are they electronically controlled (operable with one hand and not requiring tight grasping, pinching, or twisting of the wrist)? Self-closing faucets that require both reaching forward and pushing down to activate are not recommended.	 Yes  No  N/A

If a self-closing faucet is in use, does the water flow for a minimum of 10 seconds?	 Yes  No  N/A

Is the plumbing insulated or otherwise covered so that there are no sharp or abrasive edges exposed?

- Yes  No  N/A

Is there a clear floor space 30 inches wide by 48 inches long in front of the sink for a forward approach?

- Yes  No  N/A

Is the mirror mounted no higher than 40 inches to the bottom reflective edge?	 Yes  No  N/A

Is there at least one of each type of accessory (soap dispenser, towel dispenser, etc.) mounted such that the operating mechanism is no more than 48 inches above the floor?	 Yes  No  N/A

Does each accessible fixture and accessory have a clear floor space 30 inches wide by 48 inches long in front of or adjacent to the fixture or accessory?	 Yes  No  N/A

Does the toilet room have adequate space to allow a 5 foot diameter circle (to allow a wheelchair to turn 180 degrees)?	 Yes  No  N/A

Diaper changing tables are not allowed in the wheelchair accessible toilet compartment.	 Yes  No  N/A

### BATHTUB

Male		Female		Unisex		Floor or Building

- If the tub has a seat at the head end, is there a clear floor space next to the tub of 30 inches wide extending 12 inches minimum beyond the seat?	 Yes  No  N/A

- OR

- If the seat is in the tub, is there a clear floor space next to the tub of 30 inches wide extending the length of the tub?	 Yes  No  N/A

Is a seat between 17 inches and 19 inches above the bathroom door provided?	 Yes  No  N/A

Can the seat be securely placed so that it does not slip during use?	 Yes  No  N/A

Are grab bars provided as shown in the diagrams on the next page?	 Yes  No  N/A

Is the faucet control operable with one hand without requiring tight grasping, pinching or twisting of the wrist?

- Yes  No  N/A

- Does the spray unit operate both as a fixed showerhead and a handheld showerhead with a hose a minimum of 59 inches in length?	 Yes  No  N/A

### SHOWER

Male		Female		Unisex		Floor or Building

- Does the transfer shower measure 36 inches by 36 inches?	 Yes  No  N/A

- OR

- Does the roll-in shower measure at least 30 inches by 60 inches?	 Yes  No  N/A

If the shower is 36 inches by 36 inches, is there a clear floor space of 36 inches by 48 inches provided next to the shower? (The excess floor space must extend beyond the seat as in the figure above.)	 Yes  No  N/A

- OR

- If the shower is 30 inches by 60 inches, is there a clear floor space of 30 inches by 60 inches provided next to the shower? (An accessible lavatory is permitted opposite the seat.)	 Yes  No  N/A

If the alternate roll-in shower design is used (preferred option) are the inside dimensions 36 inches by 60 inches, per diagram above?	 Yes  No  N/A

Is a seat provided mounted between 17 inches and 19 inches above the bathroom floor?	 Yes  No  N/A

Is the seat a folding type in the 30 inch minimum by 60 inch minimum shower?	 Yes  No  N/A

Is the threshold at the shower no more than ½ inch in height?	 Yes  No  N/A

Are grab bars provided as shown in diagrams above?	 Yes  No  N/A

Is the faucet control reachable from the seat and operable with one hand without requiring tight grasping, pinching, or twisting of the wrist?	 Yes  No  N/A

Does the spray unit operate both as a fixed showerhead and a handheld showerhead with a hose a minimum of 59 inches in length?	 Yes  No  N/A

### BREAK ROOM

- Is the rim of the sink a maximum height of 34 inches above the floor?	 Yes  No  N/A

Is there a clear floor space at the sink positioned for a parallel approach?	 Yes  No  N/A

Do the faucets have lever handles or are they electronically controlled (operable with one hand and not requiring tight grasping, pinching, or twisting of the wrist)?	 Yes  No  N/A

Are the vending machine controls located no more than 48 inches above the floor?	 Yes  No  N/A

Are the vending machine controls easy to operate, requiring no tight grasping, pinching, or twisting at the wrist?		 Yes  No  N/A

Are at least 5% of the table top heights (but not less than 1) between 28 inches and 34 inches above the floor? (A 30-inch maximum is preferred.)	 Yes  No  N/A

Do the accessible tables have a minimum knee clearance of 27 inches above the floor that extends at least 8 inches back from the front edge? (A 29-inch height preferred; extending at least 11 inches back preferred.)

- Yes  No  N/A

Do the accessible tables have a minimum toe clearance that extends at least 17 inches beneath the table at a height of 9 inches above the floor? (24-inch minimum depth preferred.)	 Yes  No  N/A

Are the knee and toe clearances at least 30 inches in width?	 Yes  No  N/A

### AUDITORIUM

- Are wheelchair spaces provided in accordance with Table A below?	 Yes  No  N/A

#### Table A: Auditorium

| Auditorium Description | Auditorium Capacity | Wheelchair Accessible Spaces Description | # of Required Wheelchair Accessible Spaces |
| --- | --- | --- | --- |
| If Auditorium seating capacity is | 4 to 25 | The number of required wheelchair accessible spaces is | 1 |
| If Auditorium seating capacity is | 26 to 50 | The number of required wheelchair accessible spaces is | 2 |
| If Auditorium seating capacity is | 51 to 100 | The number of required wheelchair accessible spaces is | 4 |
| If Auditorium seating capacity is | 101 to 300 | The number of required wheelchair accessible spaces is | 5 |
| If Auditorium seating capacity is | 301 to 500 | The number of required wheelchair accessible spaces is | 6 |
| If Auditorium seating capacity is | 501 to 5,000 | The number of required wheelchair accessible spaces is | 6, plus 1 for each 150 (or fraction thereof) between 501 and 5,000 |
| If Auditorium seating capacity is | 5,001 and over | The number of required wheelchair accessible spaces is | 36, plus 1 for each 200 (or fraction thereof) over 5,000 |

Do the wheelchair seating spaces provided measure a minimum of 33 inches wide by 48 inches deep for a forward approach or 30 inches wide by 60 inches deep for a side approach?	 Yes  No  N/A

Is the slope of the wheelchair spaces no greater than 1:48 (2%)?	 Yes  No  N/A

Are the wheelchair seating areas designed so that companion seating is available next to the wheelchair seating?		 Yes  No  N/A

Are wheelchair spaces integrated and dispersed throughout the auditorium?	 Yes  No  N/A

Is the slope of accessible routes no greater than 1:12 (8.33%)?	 Yes  No  N/A

Is there an assistive listening system provided?	 Yes  No  N/A

Is there signage notifying the public of the availability of the assistive listening system located at the entrance to the auditorium, ticket office, and similar locations?	 Yes  No  N/A

If there are raised or lowered areas such as stages, dressing rooms, locker rooms, etc., are they accessible by a ramp or lift?	 Yes  No  N/A

### CONFERENCE ROOM

- Is the table top between 28 inches and 34 inches above the floor?	 Yes  No  N/A

Does the table have a minimum knee clearance of 27 inches above the floor that extends at least 8 inches back from the front edge?  (29-inch height preferred; extending at least 11 inches back preferred.)	 Yes  No  N/A

Does the table have a minimum toe clearance that extends at least 17 inches beneath the table at a height of 9 inches above the floor? (24-inch minimum depth preferred.)	 Yes  No  N/A

Are the knee and toe clearances at least 30 inches in width?	 Yes  No  N/A

Is there an access aisle of at least 36 inches wide provided?	 Yes  No  N/A

Is there an assistive listening system provided?	 Yes  No  N/A

Is there a mix of chairs having arm rests and casters (suggested)?	 Yes  No  N/A

### DRESSING AND FITTING ROOM

- Is there adequate clear floor space provided within the room to allow a person using a wheelchair to make a 180-degree turn (generally a 5-foot diameter circle)?	 Yes  No  N/A

With the entry door in a 90-degree open position, is there a minimum of 32 inches of clear space from the face of the door to the latch side door stop?	 Yes  No  N/A

Is the door hardware operable by a single effort with one hand not requiring tight grasping, pinching, or twisting of the wrist?	 Yes  No  N/A

Does the room provide a fixed bench 17 inches to 19 inches above the floor that is between 20 inches to 24 inches deep and 42 inches minimum in length?	 Yes  No  N/A

Does the bench have a back rest (a wall can serve as a back rest)?	 Yes  No  N/A

Do 5% - but not less than one – of the dressing rooms for each type of use in each cluster of dressing rooms comply with the requirements in this section?	 Yes  No  N/A

## ELEMENTS

### TELEPHONE

- Are all public telephones equipped with volume control?	 Yes  No  N/A

Are the telephone controls pushbutton type?	 Yes  No  N/A

Is there a clear floor space of at least 30 inches by 48 inches adjacent to at least one telephone per floor?

- Yes  No  N/A

Are the operating controls on accessible phones located no more than 48 inches above the floor?

- Yes  No  N/A

Is the cord to the handset on accessible telephones at least 29 inches long?	 Yes  No  N/A

If there are four or more public telephones, is at least one fixed text telephone (TTY) provided?

- Yes  No  N/A

Do the telephone locations not providing a TTY have directional signage indicating the location of the nearest TTY device?	 Yes  No  N/A

Are the accessible telephones integrated and dispersed throughout the floor?	 Yes  No  N/A

### DRINKING FOUNTAIN

- Is there a clear floor space of at least 30 inches by 48 inches provided for a forward approach to the drinking fountain? (Fountains without knee clearance can provide a parallel approach.)	 Yes  No  N/A

Is a knee clearance at least 27 inches above the floor provided at the front edge that extends back from the front edge at least 8 inches?	 Yes  No  N/A

Is the spout mounted at the front of the unit?	 Yes  No  N/A

Is the spout mounted no more than 36 inches above the floor?	 Yes  No  N/A

Is the operating control mounted at the front of the unit or on the side near the front of the floor?

- Yes  No  N/A

Is the water flow at least 4 inches high in a trajectory parallel or nearly parallel to the front of unit?

- Yes  No  N/A

Do 50% of the drinking fountains per floor comply with questions 1-6 above?	 Yes  No  N/A

Do 50% of the drinking fountains per floor have a spout height between 38 inches and 43 inches above the floor?

- Yes  No  N/A

If there is only one drinking fountain on the floor, does it have both a high and a low spout?	 Yes  No  N/A

Does the drinking fountain protrude more than 4 inches from the wall between 27 inches and 80 inches above the floor?	 Yes  No  N/A

### INFORMATION KIOSK AND AUTOMATED TELLER MACHINE

- Is a clear floor space 30 inches wide by 48 inches long provided next to the kiosk?	 Yes  No  N/A

Are the controls located no more than 48 inches above the floor?	 Yes  No  N/A

Are all operating controls and mechanisms operable with one hand without tight grasping, pinching, or twisting of the wrist?	 Yes  No  N/A

Do all operating controls and mechanisms operate with no more than 5 lbs. of force?	 Yes  No  N/A

Are all instructions and information accessible to and independently usable by persons who are blind or have limited vision?	 Yes  No  N/A

### SALES AND SERVICE COUNTERS

- Do sales counters and counters for distribution of goods or services have a portion of the counter at least 36 inches wide that is no more than 36 inches above the floor? (does not apply to check-out aisles)	 Yes  No  N/A

Are accessible counters dispersed throughout the facility?	 Yes  No  N/A

Are the accessible counters located on accessible routes?	 Yes  No  N/A

Are accessible check-out aisles identified with the international wheelchair symbol mounted above the check-out aisle in the same location as the check-out number or type of check-out?	 Yes  No  N/A

Does the number of accessible check-out aisles provided comply with Table B below?	 Yes  No  N/A

#### Table B: Sales and Service Counters

| Check-out Description | Number of each function | Accessible Check-out Description | # of Required Accessible Checkout Aisles |
| --- | --- | --- | --- |
| If the total number of check-out aisles of each function is | 1 to 4 | The minimum number of accessible check-out aisles must be | 1 |
| If the total number of check-out aisles of each function is | 5 to 8 | The minimum number of accessible check-out aisles must be | 2 |
| If the total number of check-out aisles of each function is | 9 to 15 | The minimum number of accessible check-out aisles must be | 3 |
| If the total number of check-out aisles of each function is | More than 15 | The minimum number of accessible check-out aisles must be | 3, plus 20% of additional aisles |

## SPECIAL FACILITIES

### RESTAURANT AND CAFETERIA

(These requirements are in addition to all other applicable requirements.)

- Are dining surface heights (tables and counters) between 28 inches and 34 inches above the floor (30 inch maximum height preferred)?	 Yes  No  N/A

Do dining surfaces have a minimum knee clearance of 27 inches above the floor (29 inches preferred)?

- Yes  No  N/A

Does the knee clearance extend at least 8 inches beneath the dining surface (11 inches minimum preferred)?

- Yes  No  N/A

Does the dining surface have a minimum toe clearance that extends at least 17 inches beneath the dining surface at a height of 9 inches above the floor?	 Yes  No  N/A

Are the knee and toe clearances at least 30 inches in width?	 Yes  No  N/A

Are accessible dining surfaces located on an accessible route that is a minimum of 36 inches wide?

- Yes  No  N/A

Are a minimum of 5% of the dining surfaces in compliance with questions 1-6 above?	 Yes  No  N/A

Are all dining areas, including raised and sunken areas and outdoor areas, located on an accessible route? (In non-elevator buildings, mezzanine seating areas that provide less than 25% of the total dining seating are not required to be located on an accessible route.)	 Yes  No  N/A

Are tray slides no more than 34 inches above the floor?	 Yes  No  N/A

Does the food service line have a minimum clear width of 36 inches (42-inch preferred)?	 Yes  No  N/A

Are the condiments and tableware located within the reach ranges shown in the diagrams below?

- Yes  No  N/A

Are the vending machine controls easy to operate requiring no tight grasping, pinching, or twisting of the wrist?

- Yes  No  N/A

- Are the vending controls located no more than 48 inches above the floor?	 Yes  No  N/A

### LIBRARY

(These requirements are in addition to all other applicable requirements.)

- Are table top heights between 28 inches and 34 inches above floor (a 30-inch maximum preferred)?

- Yes  No  N/A

Do the same tables have a minimum knee clearance of 27 inches above the floor (29-inch preferred)?

- Yes  No  N/A

Does the knee clearance extend at least 8 inches beneath the table (11 inches minimum preferred)?

- Yes  No  N/A

Does the table have a minimum toe clearance that extends at least 17 inches beneath the table at a height of 9 inches above the floor?	 Yes  No  N/A

Are the knee and toe clearances at least 30 inches in width?	 Yes  No  N/A

Are a minimum of 5% of each type of fixed seating, tables, or carrels in compliance with questions 1-5 above?

- Yes  No  N/A

Do 5% (not less than one) of the check-out areas have a minimum counter area measuring 36 inches wide which is no more than 36 inches above the floor?	 Yes  No  N/A

Is there a minimum clear space of 36 inches between book racks, card catalogs, magazine displays, etc.?

- Yes  No  N/A

Are card catalogs and magazine displays located 48 inches maximum above the floor?	 Yes  No  N/A

### MEDICAL CARE FACILITY

(These requirements are in addition to all other applicable requirements.)

- In general purpose hospitals, psychiatric facilities and detoxification facilities, are at least 10% of the patient bedrooms and associated toilet rooms accessible?	 Yes  No  N/A

In hospitals and rehabilitation facilities specializing in mobility treatment, are all patient bedrooms and associated toilet rooms accessible?	 Yes  No  N/A

In long-term care facilities and nursing homes, are at least 50% of the patient bedrooms and associated toilet rooms accessible? (ADA requirement)	 Yes  No  N/A

In long-term care facilities and nursing homes, are the non-accessible patient rooms adaptable (provide wheelchair maneuvering space and clearances without actual accessible fixtures, such as grab bars, knee clearances, etc.)?

- Yes  No  N/A

Does each accessible patient room provide a turning space to allow a person using a wheelchair to make a 180-degree turn? (In rooms with two beds, it is preferred that the space be provided between the two beds.)

- Yes  No  N/A

Does each accessible patient room provide a 36-inch minimum wide space on each side of the bed that is connected to an accessible route?	 Yes  No  N/A

Are bathrooms associated with the accessible bedroom accessible?	 Yes  No  N/A

If the medical care facility is an outpatient unit, are at least 10% of the parking spaces accessible and reserved for persons with disabilities?	 Yes  No  N/A

- OR

- If the medical care facility specializes in treatment or services for persons with mobility impairments, are at least 20% of the parking spaces accessible and reserved for persons with disabilities?	 Yes  No  N/A

When telephones are located in or adjacent to a hospital emergency room, recovery room, or waiting room, is there a text telephone (TTY) available at each such location?	 Yes  No  N/A

### TRANSIENT LODGING

(These requirements are in addition to all other applicable requirements.)

- Is each accessible unit, sleeping room, or suite located on an accessible route?	 Yes  No  N/A

Do the accessible sleeping rooms have a 36-inch clear width maneuvering space on each side of a single bed?

- Yes  No  N/A

- OR

- If there are two beds, is the 36-inch space provided between the beds?	 Yes  No  N/A

Do all doors in common use areas and within each accessible unit, sleeping room, or suite comply with the requirements contained in the BUILDING: DOORWAY section on pages 26 through 28 of this survey? (Doors not requiring full user passage, such as shallow closets, may have a clear opening not less than 20 in.)

- Yes  No  N/A

Do all doorways within the non-accessible sleeping rooms and suites provide a clear opening of 32 inches measured from the face of the open door to the latch side door stop? (Doors not requiring full user passage, such as shallow closets, may have a clear opening not less than 20 inches.)	 Yes  No  N/A

Is at least one of each type storage space within the accessible unit, sleeping room, or suite no higher than 48 inches above the floor?	 Yes  No  N/A

Are all controls within the accessible unit, sleeping room, or suite no higher than 48 inches above floor?

- Yes  No  N/A

When provided as part of an accessible unit, sleeping room, or suite, are the following spaces accessible and located on an accessible route:

- The dining area?	 Yes  No  N/A

- The living area?	 Yes  No  N/A

- At least one sleeping area?	 Yes  No  N/A

- Patios, terraces, or balconies?	 Yes  No  N/A

- At least one bathroom?	 Yes  No  N/A

Is at least 50% of the shelf space in cabinets no more than 48 inches above the floor?	 Yes  No  N/A

Is at least 50% of the shelf space in refrigerators/freezers no more than 54 inches above the floor?

- Yes  No  N/A

Is the number of accessible units, sleeping rooms, or suites in compliance with Table C below?	 Yes  No  N/A

Is a visual alarm provided which is connected to the building emergency alarm system?	 Yes  No  N/A

Is the alarm visible from all areas within the unit, room, or suite?	 Yes  No  N/A

Is a visual notification system provided for incoming telephone calls and a door knock or bell?	 Yes  No  N/A

If telephones are provided in the unit, room, or suite, do they have volume controls?	 Yes  No  N/A

Is there an electrical outlet provided within 4 feet of the telephone to facilitate the use of a TTY?

- Yes  No  N/A

Is the number of accessible units, sleeping rooms, or suites meeting the requirements of questions 11-15 in compliance with Table D below?	 Yes  No  N/A

At least one, but not more than 10%, of the rooms required to comply with Table B must be provided in the rooms complying with Table C.

#### Table C: Transient Lodging (1)

| Total Number of Rooms | Required Number of Roll-in Showers | Required Number of Accessible Rooms |
| --- | --- | --- |
| If the total number of rooms is 1 to 25 | None | At least 1 room must be accessible |
| If the total number of rooms is 26 to 50 | None | At least 2 rooms must be accessible |
| If the total number of rooms is 51 to 75 | At least 1 room must have a roll-in shower AND | The total number of accessible rooms must be at least 4 |
| If the total number of rooms is 76 to 100 | At least 1 room must have a roll-in shower AND | The total number of accessible rooms must be at least 5 |
| If the total number of rooms is 101 to 150 | At least 2 rooms must have a roll-in shower AND | The total number of accessible rooms must be at least 7 |
| If the total number of rooms is 151 to 200 | At least 2 rooms must have a roll-in shower AND | The total number of accessible rooms must be at least 8 |
| If the total number of rooms is 201 to 300 | At least 3 rooms must have a roll-in shower AND | The total number of accessible rooms must be at least 10 |
| If the total number of rooms is 301 to 400 | At least 4 rooms must have a roll-in shower AND | The total number of accessible rooms must be at least 12 |
| If the total number of rooms is 401 to 500 | At least 4 rooms must have a roll-in shower AND | The total number of accessible rooms must be at least 13 |
| If the total number of rooms is 501 to 1,000 | At least 1% of the total must have a roll-in shower AND | At least 3% of the total rooms must be accessible |
| If the total number of rooms is 1,001 and over | At least 10, plus 1 for each additional 100 over 1,000 must have a roll-in shower AND | At least 30, plus 2 for each additional 100 over 1,000, must be accessible |

Example (for buildings with 1,001 or more rooms):

A building with 1,200 total rooms would need to have—

12 rooms with roll-in showers: 10, plus 1 for each hundred of the 200 rooms over the 1,000-room mark.

34 accessible rooms: 30, plus 2 for each hundred of the 200 rooms over the 1,000-room mark.

#### Table D: Transient Lodging (2)

| Number of Rooms | Accessible Rooms |
| --- | --- |
| If the total number of rooms is 1 | The minimum number of accessible rooms must be 1 |
| If the total number of rooms is 2 to 25 | The minimum number of accessible rooms must be 2 |
| If the total number of rooms is 26 to 50 | The minimum number of accessible rooms must be 4 |
| If the total number of rooms is 51 to 75 | The minimum number of accessible rooms must be 7 |
| If the total number of rooms is 76 to 100 | The minimum number of accessible rooms must be 9 |
| If the total number of rooms is 101 to 150 | The minimum number of accessible rooms must be 12 |
| If the total number of rooms is 151 to 200 | The minimum number of accessible rooms must be 14 |
| If the total number of rooms is 201 to 300 | The minimum number of accessible rooms must be 17 |
| If the total number of rooms is 301 to 400 | The minimum number of accessible rooms must be 20 |
| If the total number of rooms is 401 to 500 | The minimum number of accessible rooms must be 22 |
| If the total number of rooms is 501 to 1,000 | The minimum number of accessible rooms must be 5% of Total |
| If the total number of rooms is 1,001 and over | The minimum number of accessible rooms must be 50, plus 3 for each 100 (or fraction thereof) over 1,000 |

Example (for buildings with 1,001 or more rooms):

A building with 1,200 total rooms would need to have:

56 accessible rooms (50, plus 3 for each hundred of the 200 rooms over the 1,000-room mark).

### CHILDREN’S ENVIRONMENTS

Special Note: If children’s fixtures are provided, they must comply with the criteria outlined in this section.

#### Drinking Fountains – For children ages 12 and younger

- Is the spout height no more than 30 inches above the floor?	 Yes  No  N/A

Is a parallel approach provided adjacent to the drinking fountain?	 Yes  No  N/A

#### Restrooms – For children ages 12 and younger

Is the height of the toilet seat between 11 inches and 17 inches above the floor?	 Yes  No  N/A

Is the height of the horizontal grab bar between 18 inches and 27 inches above the floor?	 Yes  No  N/A

Is the distance from the centerline of the toilet to the side wall between 12 inches and 18 inches?

- Yes  No  N/A

Is the clear floor space for the toilet at least 5 feet wide (toilet within this space)?	 Yes  No  N/A

Is there at least 48 inches of clear floor space in front of the toilet?	 Yes  No  N/A

- OR

- Is the depth of the clear floor space, measured from the wall behind the toilet, at least 78 inches?

- Yes  No  N/A

Is the toilet paper dispenser mounted below the horizontal grab bar at least 1 inch above the seat?

- Yes  No  N/A

Are the flush valves for the toilet no more than 36 inches above the floor?	 Yes  No  N/A

For ages 6-12, is the rim of the sink no more than 31 inches above the floor?	 Yes  No  N/A

For ages 6-12, is the knee clearance at least 24 inches from the floor to the bottom of the apron?

- Yes  No  N/A

- For ages 5 and younger, rim height and knee clearance do not apply, but is a parallel approach provided adjacent to the sink?	 Yes  No  N/A